- DESCRIPTION OF WORK

COMPANY wishes to acquire the services of  EPC Contract to provide Engineering, Procurement, Construction, commissioning and start-up of ARAB GAS PIPELINE PROJECT - PHASE II and associated facilities as specified in the EXIBIT-B, Technical section of this RFQ.  The WORK to be included in the CONTRACT embraces engineering, basic/detailed design, procurement, delivery of materials and equipment, construction, equipment installation, pre commissioning, commissioning, training of COMPANY's  staff, documentation and guarantee/warranty of the project.

Complete Facilities of ARAB GAS PIPELINE PROJECT - PHASE II to be constructed, installed and commissioned by CONTRACTOR in Syria as Turn-key based job.

PROJECT DESCRIPTION

ARAB GAS PIPELINE PROJECT - PHASE II shall be performed at the facilities located at different area, starting from AL Foreqls area (about 50 km east of Homs City) at BVS 6011 where A Fiscal Metering Station (FMS) including  Gas Compressor units will be installed .

This PROJECT shall mainly consist of the following activities:

Engineering, Procurement, Construction and Installation of the 36" Gas Pipeline System between ARAB GAS PIPELINE PROJECT - PHASE I  36” flange located at  BVS-6011 ( AL Foreqls area) and  ARAB GAS PIPELINE PROJECT - PHASE II-PART I  36” flange located at R-7003  near Aleppo PRMS, Four  Block Valve Stations, One Intermediate Station, One FMS  at  BVS-6011 ( AL Foreqls area) including  Gas Compressor units, Two Pig Launcher and Two Pig Receiver and Four tie-in as follow:

-with ARAB GAS PIPELINE PROJECT - PHASE I  36” flange. at  BVS-6011.

-with ARAB GAS PIPELINE PROJECT - PHASE II-PART I  36” flange. at R-7003.

-with 36" inlet/outlet of the FMS & gas compressor.

with all related piping, materials, equipment, instrumentation, electrical, control , SCADA and telecommunication systems, civil and Cathodic protection system…etc. to hand over safely operable project as per RFQ documents, international codes, standards and Specification and according to the Data sheets, Drawing, Codes and Standards and AFPC Schedules with all associated piping, instrumentation, civil, electrical, control and monitoring system, DCS system, SCADA and telecommunication systems , accessories, lubrication, special tools, spare  parts, consumable, warranty and guarantee, testing and documentation. As further described in the RFQ

AL FOREQLS FISCAL METERING STATION (FMS)

The contractor’s scope of work. At the FMS  shall include, but not limited to, all Civil and structural works such as: equipment, skids foundations, pipe sleepers/pipe supports, leveling, fencing, painting etc… as per this RFQ specifications, cods, standards and documentation's. The detailed scope of work should be shown on the contractor supplied drawings.

Contractor’s scope of work at the FMS, includes, but not limited to the following:

The area to be prepared and fenced shall be of 20000m2 as minimum..

The following building types shall be constructed at the FMS location:

I-Transformer room

II-Control Building: area not less than 400m2 , consists of the following:

-Control room

-4 Offices + Meeting office

-MCC/Switch gear + UPS

-Battery room

-Buffet room

-Utilities.

III-Workshop to be not less than 200 m2 consists of  the following:

-Workshop room  7m high with 15 ton overhead crane.

-4 Offices

-Library office with required cabinet and shelves

-Buffet room

-Utilities.

IV-Guardhouse: See attached Drawing

Construct and install steel structural Hangar for the gas compressors.

Supply and install potable water system using required (2”) galvanized pipe and distribute the water to all skids units, buildings etc. this shall include the supply and installation of (30M3) water storage tank with required pumps and has a level 10m.

Design and install sewage drain system with leaching fields or septic tanks.

The skid drains shall be routed to a closed drain tank equipped with safety equipment and vacuum pump

Access Asphalt road

Asphalt yards should be approved by SGC, and concrete yards around over all equipment skids area for width not less than 2m.

Sun shade car parking for 10 cars.

CONTRACTOR shall design, procure, deliver and install as further described in this RFQ documents, Exhibit-B Technical as minimum the following for each FMS :

-metering trains skid with filters

-fuel/instrument gas skid.

-Cyclon Separator.

-Stand-by diesel generator

-DCS Control  and monitoring System

-Gas Detection  & Fire Fighting System

-SCADA &Telecommunication System

-Blow down / flare system

-Utility/Drain systems.

Buildings, access roads and any other material and equipment necessary to complete the project

and all associated piping, valves, instrumentation, civil work, fencing, electrical and spare parts, commissioning and start-up..etc.

Block Valve Stations (BVS):

-	Design, procure, deliver and insatallation of Four (4) Block Valve Stations (BVS), along

the pipeline, with all associated Buildings and facilities.

Pipeline:

Design. Procure, excavation,  Laying, Welding ,coating,  lowering and backing , cleaning and hydrotesting / drying ,cathodic protection, and commissioning of approximately 186 km of 36'' API 5L X70 pipeline .

Pig Launchers/Receivers:

Contractor shall design, procure, deliver and install:

- ONE (1) Pig Launchers at ALFOREQLS FMS .

- ONE (1) Pig Receivers at R-7003 IN Aleppo PRMS.

- ONE (1) Pig Launchers at Intermediate station .

- ONE (1) Pig Receivers at Intermediate station ..

Four  Tie-in with existing network as described in EXHIBIT B.

SAFETY OBJECTIVES:

Recognized codes, standards and procedures for the design and construction of the plant and specifications of equipment shall be applied and complied with:

A target of zero lost time incident during the construction and commissioning phase.

Safety critical activities and hazards are to be identified during the bid period and safety management system to achieve the above objectives together with management system to quality assure safety activities are to be submitted as a part of  Bidders bid.

Environmental Impact Assessment ( EIA )

Environmental Impact of the projet should be assessed in a comprehensive integrated manner via a systematic examination of unintended consequenes of the projet most phases , with the view to reduce or mitigate negative consequenes and capitalize on positive ones , the Environmental Impact Assessment( EIA )Study should be intended to the competent authorities for approval before commencing contruction activites of the project .

The Study should comply with the applicable regional , local , internationl guidelines of environmental impact assessment such as:

1.  Syrian environmental regulations.

2.  World bank EIA guidelines.

3.   Project owners future requirements.

4.   Local and municipal requirements over the pipeline route.

The EIA study shoud prove the following:

- To ensure the protection and conservation of the environment and natural resources including human health aspects against uncontrolled operations.

- To ensure a sustainable economic development that meets the present needs without compromising future generation's ability to meet their needs.

- Provide baseline information for later comparative purposes.

- Demonstrate a capability of self-monitoring and regulaion, and providing data which can assist in debate with regulatory authorities.

- Provide assurance to authorities and the public thereby generating trust and confidence.

- Demonstrate a model of scientific and technical credibility when participating in local debate.

- Minimize exposure and prevent liabilities.

- Enhance the Company image.

The study report should address the following items in details:

- Executive Summary

- Project Description

- Description Of The Environment

- Effect Of Projects On Environment

- Environmental Impact Assessment

- Mitigation Measures

- Monitoring Plans

- Project Alternatives

- Non-Technical Summary

- Recommendation

The contactor will be responsible for the study most stages till the study full approval from the concerned competent authorities that may include:

- Data and information gathering.

- Testing, examinations, analysis required for the study.

- Consultations, advice, and other services needed from any scientific institutions, consultants....etc.

- Study reports issuance and printing for the concerned parties.

- Clarifications or extra assessments needed form the authorities review the study including any of the abovementioned activities.

- Reports, statements, and monitoring reports needed within the construction activities of the project.

USE OF SKID MOUNTED EQUIPMENT:

Equipment shall be installed on skids. Each of skid shall be designed to accommodate the maximum level of related valving and instrumentation and ensure the sufficient maintenance space within the skid and to minimize the hook-up between skids.

All facilities, major equipment and components thereof, shall be arranged on the skids so that easy access by personnel, and if applicable, carnage is always insured.

The pipe work and instrument arrangements are to allow that each piece of equipment can be serviced and  removed for maintenance.

STANDARDIZATION

The specification and selection of equipment and materials will be done in such a manner that the variety of manufacturers and types is reduced where possible.  The aim is to select a single manufacture for similar groups of equipment, such as electric motors, pumps, instruments, mechanical seals, etc.  These items will be , as and where possible, in accordance with equipment

already in use by COMPANY in its existing gas facilities. All equipment selected must be of simple and robust construction and shall be designed as per specification, codes and standards specified in, but not limited to, the “Exhibit-F, Data Reference and CONTRACT’S GENERAL SPECIFICATIONS”.