SPARE PARTS DOCUMENTATION AND PURCHASING

The action required completing the SPARE form and its continuation sheet, if necessary. Is described in the following parts:

PART 1 covers sections to be completed by the contractor prior to issue of SPARE form.

PART 2 covers columns 1 to 12 inclusive, 19 and 20. it described information to be provided by the equipment manufacturer ( vendor)

PART 3 covers columns 13,14,15,18 and block 21. it describes the action to be taken either individually or jointly by the contractor and operating division of company ( SGC) .

PART 4 covers columns 16, 17 and authority block 22. It describes the action to be taken by the party/parties usually nominated in the Project Specification, normally Operation Division.

1.DETAILED INSTRUCTIONS FOR COMPLETION OF PART 1

It is recommended that the contractor prior to submission to vendor complete the section .

SPIR NO.: This number will be allocated by Contractor on a sequential basis as follows:

Mechanical Equipment- S100 Onwards

Electrical Equipment    -S500 Onwards

Instrumentation            -S700 Onwards

Subject:                Brief  description of equipment

Manufacturer:        Name of prime manufacturer (i.e. manufacturer with whom Purchase Order

is placed)(Vendor).

Date:                     Date of issue of SPIR Form to Vendor.

Project/Plant:         Name of project to which equipment on SPIR Form refers.

Consignee:             Name of Company e.g. Syrian Gas Company.

Engineering by:       Name of Contractor undertaking procurement of spares.

2. DETAILED INSTRUCTIONS FOR COMPILATION OF PART 2.

To be completed by the equipment manufacturer in the following sequence:

Column 1- Enter the same equipment registration number(s) or tag number(s) for each piece of

equipment as stated in requisitions and/ or purchase orders.(Tag Nos.are much

preferred)

Column 2- Manufacturers model, type or other positive identification reference of  the equipment

Ordered

Column 3-  Manufacturers serial number or other control reference of the equipment/instruments as

being supplied

Column 4-  Total number of pieces of identical equipment listed in columns 1 to 3 and 6.

Column 6-  Purchasing Company's order reference number.

Column 8-  Describe all parts in English language. List all parts, which should be carried in stock

For normal operation and also list slow-wearing parts.

If an item is interchangeable between two or more units it should be listed once only.

Column 9-   for each part entered in Column 8 enter the drawing number or reference number.

Note: The Manufacturer should attach these drawings to the SPIR.

Column 10- Enter the reference numbers/letters or other information which identifies each part.

Interchangeability with identical parts within the Manufacturers range should be

Indicated.

Owing to the wide variety of systems in use for identification of parts, it is impossible t

lay down firm rules for completing these columns .

Manufacturers should give whatever identification system they use to positively

Identify parts and to show interchangeability with existing equipment.

Manufacturers final cross-sectional drawings, workshop drawings and real part

numbers may not always be available in the early stage of manufactures. This may

cause delays in timely preparation of manufacturers recommendation for initial spare

parts in column 12 and subsequent ordering of same.

If delays are unacceptable, it is recommended that reference is made to

Manufacturer's recommendation, which is readily available such as pamphlets,

brochures, exploded views, typical drawings of similar equipment.

- manufacturers are advised that "initial spare parts" are defined as follow:

"Initial spare parts are safeguard the operation of equipment during the running in and starting up period, and the quantity should be adequate to cover the period until the arrival of spare parts for normal operation.

Latter period should for practical purposed be taken as approximately one year.

Initial spare parts must therefore be ordered at such an early date that ideally they will arrive at site at the same time as the parent equipment, but in any case prior to start up."

Column 11-     Enter material specifications in term of full international standards and accepted

Conventions, not manufacturers or sub- manufacturer's references.

Column 5-       For each equipment or group of identical equipment, enter in the appropriate space

Opposite each part the quantity of the described parts fitted in each piece of

equipment or instrumentation.

Column 7-        Enter the total number of identical parts in all equipment specified. In the case of

identical units multiply the quantity in Column 5 by the number of units given in

Column 4.

Column 12-      Enter the manufacturers recommended quantities of initial spare which are likely

to be needed during start up and the first year of normal operation.

Column 19-      Enter the approximate ex- works price per piece of each of each part in the

currency shown at the top of column.

Column 20-      Enter the original manufacturers identification numbers of all items of third party

manufacture "bought out" items such as ball bearings, oil and mechanical seals,

couplings.

Note:                 Manufacturers must not complete Columns 13 to 18 inclusive nor Blocks 21

and 22.

3-DETAILED INSTRUCTIONS FOR COMPLETION OF PART 3

It is recommended that the columns are completed by the contractor and/or the Operation

Division of Company in the following sequence:

Column 18          For instrumentation this column should be left back. Other equipment and its

Spare parts should be classified in accordance with the guidance given in the

attached ANNEX-1.

Equipment classification

Equipment meeting the" vital" definition should be coded by V.

Equipment meeting the" essential" definition should be coded by E.

Equipment meeting the" auxiliary" definition should be coded by A.

Spare parts classification

Parts identified "consumable" should be coded C.

Parts identified "replacement" should be coded R.

Parts identified "insurance items" should be coded I.

Column 13 * (for initial spares only):

Having duly scrutinized the manufacturers recommendation in column 12, enter the quantities of initial spare parts recommended .

Although primarily intended to cover the start up period and early stages of normal operation, the quantities entered here should be integrated with the parts for normal operation i.e. Column 15 will represent the total recommended number of spare parts to be ordered .

Column 14*      (for normal spares only):

If applicable to the project, enter the quantities of spare parts for normal operation.

Column 15*      (for normal spares only):

In column 15, the Operation Division finally recommended the quantity of each

part listed under Column 8, which is required for start up and normal operation

taking into account the existing stock position and consumption history.

For instrument parts Column 15 will indicate the quantities required as initial spare

parts and those required for the early stage of operation.

- To avoid any misunderstanding by users of the completed SPIR forms, it is

emphasized that the quantities calculated and entered in Columns 13, 14 and 15 must not be ordered, since they represent only recommendations/ technical approval.

Note: Purchasing must not proceed until the authority to order is given by the party who

controls the project budget expenditure. This authority must explicitly be given by

completing Block 22, see Part 3.

Block 21-    Enter the day by, which spare parts must be available for use on site.

Generally known as the "required on site" date, this should be consistent with the

actual and latest known start up and commissioning period as advised by the project

leader.

Note: At this stage it will be helpful for users to know that the form now contains final

recommendations. This fact should be clearly indicated, as should the date of

completion.

4-DETAILED INSTRUCTIONS FOR COMPLETION OF PART 4

It is recommended that the parties indicated below to complete the columns:

Column 16-  The parties for authorizing expenditure (or his delegate) enters in this Column the

exact quantities to be ordered.

Authority Block 22

This block must give the name of the Company or other authority who is

Authorizing the purchase of the quantities entered in Column 16.

Column 17- This Column should be left blank.