EXH-F	Appendices and Special Project Specifications

SPS-1	Specification of Line Break Detection Unit

SCHEDULE SPS-1

PROJECT SPECIFICATION FOR LINE BREAK DETECTION UNIT

TABLE OF CONTENTS

1.0	SCOPE OF DEFINITIONS.

2.0	REFERENCE CODES AND STANDARDS.

3.0	DESIGN DATA

4.0	SYSTEM DESIGN REQUIREMENTS.

5.0	SYSTEM MANUFACTURING REQUIREMENTS.

6.0	MARKING AND SHIPMENT.

7.0	VENDOR DOCUMENTATION AND DATA REQUIREMENTS.

8.0	SPARE  PARTS REQUIREMENTS.

1.0	SCOPE:

This project specification covers the requirements for:

Line Break detection unit consisting of at least a pneumatic to open and spring to close actuator with manual override suitable to operate a soft-seated, carbon steel, full bore, (36 inch nominal OD 600 1bs) ball valve, complete with sensors, transmitters and all other controllers and equipment as required for automatic line break detection and control service.

Each unit shall be assembled and tested with the ball valve, to form a completely assembled self contained unit suitable for service and environmental conditions detailed in this Specification.

Line Break detection control units shall be installed within block valve stations on the main 36” pipeline where indicated on Company’s drawings or prescribed in the Scope of Work.

A portable line break detection testing device, to adjust in the field the setting of the differential pressure rate of the unit within the tolerances specified. This testing device shall become property of Company.

2.0	REFERENCE CODES AND STANDARDS

The unit shall be designed as per RFQ data and specifications and internation codes and standards but not limited to that provided in the “Exhibit-G, Data References and Contract’s General Specifications”.

3.0	DESIGN DATA

| Medium | ### Dry Gas |
| --- | --- |
| Operator | Spring return, pressure to open, with line break detection unit and manual override. |
| Pipeline Min. Operating Press | 30 Barg |
| Pipeline Max. operating Press | 72 Barg |
| Pipeline Design Pressure | 75 Barg |
| Closing time of Valve | As per standards |
| Maximum Air Temperature | 52 degree C |
| Minimum Air Temperature | -10 degree C |
| Maximum black bulb temperature | 82 degree C |

Gas Composition:

As per specifications provided in Exhibit-G, Data References and Contract’s General Specifications.

4.0	SYSTEM DESIGN REQUIREMENTS

4.1	The minimum torque produced by the actuator shall be:

- 	At least 1.9 times the torque required to operate a new ball valve under the worst operating conditions with vented body cavity at a differential pressure equal to the pipeline design pressure.

-	At least 1.5 times the amount required to operate the valve after a long period of non-operation at the same conditions as indicated above.

4.2	Valve and actuator shall be suitable for installation with the valve stem in vertical up direction.

The actuator shall be of lubrication free design.

The actuator shall be entirely weatherproof and the construction shall be such that its performance shall be in no way impaired by corrosion or other kinds of ingress.

Moreover, the construction shall be “vandal-proof” and suitable for desert conditions.

4.4	The equipment shall be suitable for natural gas tapped from the pipeline upstream of the valve, as power supply and signal fluid.

Ambient condition, pressure and temperature fluctuation shall not impair the proper functioning of the actuator.

4.5	The actuator shall have pre-set, adjustable end stops of sufficient size to absorb the maximum torque output.

Viewed from the valve top, the closing direction shall be clockwise.

Valve position shall be shown by indicator and shall be equipped with limit switches for valve status data transmission to control room.

4.6	The space between the actuator and the valve shall be completely enclosed, either by a separate spool piece or by the valve or actuator body.

The center body section shall be fully enclosed to provide protection from all moving parts and eliminate the possibility of injury to personnel.

The valves shall be fully buried, with extended spindles; protective covering shall be supplied suitable for this arrangement.

4.7	The actuator shall be provided with a manual mechanical override with bevel gear reduction units.

The hand wheel for manual operation shall have no protruding spooks.

4.8	All materials for pressure retaining parts shall be traceable to source and shall be appropriately certified.

4.9	All structural and pressure retaining elements shall be designed with equal or higher safety factors than those for the valve, based on maximum loads that may occur during operation.

The enclosure shall have suitable bearing surfaces to absorb side loading that could be transmitted to the valve stem.

4.10	The actuator control shall contain all features and accessories for a self-contained unit to fulfil automatic line break detection.

The power supply system shall be provided with external connection portable power supply equipment.

Moreover, a lock up device (lock up tank facilities) shall be provided to prevent the valve from closing at insufficient pipeline pressure.

A second gas bottle shall create the rate of pressure drop in combination with the adjustable orifice. This bottle shall not act as a second back-up reservoir.

4.11	The testing device shall include all the features and accessories to carry out Line Break detection unit field adjustment as a portable self-contained control.

4.12	The dimensions of the actuator with mechanical manual override shall be within the maximum dimensions of the block valve stations so as to allow removing via the removable roof panels.

5.0	SYSTEM MANUFACTURING REQUIREMENT

5.1 	Inspection and Testing, General

Each actuator shall be tested as set out in the following paragraphs:

Materials subject to inspection shall be clean and oil free.

The Contractor shall conduct all acceptance tests at the same time, the results of all tests shall be recorded.

5.2 	Inspection

All actuators and testing device shall be subjected to visual inspection.

Smooth and even operation shall be required. The position of the output shaft shall be monitored during running of the test. Any deformation, misalignment, damage, eccentricity etc, outside previously approved specifications shall cause rejection.

5.3	Cylinder Pressure Test

1 hour.

The pressure test shall be recorded and the actuator shall thereafter be visually inspected for defects.

Visible leaks, deformations etc shall cause rejection.

5.4	Performance Test

The output of the actuator shall be determined in accordance with a previously approved procedure. The torque output throughout the stroke and changes in relation to the operating pressure shall be recorded.

5.5	Functional Test

The valve and actuator shall be assembled to be a complete unit with all necessary auxiliaries prior to final inspection and shipment.

All complete unit shall be visually inspected for workmanship and compliance with requisition, and functionally tested for correct operation at the calibrated settings of the sensing devices, with energized equipment.

6.0	MARKING AND TRANSMPORTATION:

6.1 	Marking.

The actuator and device shall have a permanently attached name plate, showing the following details:

-	Manufacturers name or trademark.

-	Manufacturers serial number.

-	Model or type.

-	Direction of rotation.

-	Maximum working pressure.

-	Maximum torque output.

-	Type of fluid / lubricant.

The first 4 items shall also be legibly indicated on the actuator body by either low stress stamps, or by cast in marks on the actuator body.

6.2	Shipment:

If required, for safe shipment and to protect material against damage, the actuator and valve shall be packed separately. Where required, extra gaskets shall be supplied.

The CONTRACTOR shall be responsible for the installation of plugs or other adequate sealing to prevent subsequent ingress of moisture and dirt.

7.0	VENDOR DOCUMENTATION AND DATA REQUIREMENTS:

7.1	Operating Manual:

The CONTRACTOR shall be responsible for the providing of an Operating Manual for Company’s personnel who will be responsible for the safe and correct testing, adjustment and operating manuals of these actuators. The manuals will be completely in English and will at least cover:

-	A full schematic diagram and unit drawing.

-	Installation instructions.

-	Full description of how to operate testing device.

-	Description of how to adjust the setting on the actuator.

- 	Description of maintenance requirement for the actuators and testers.

Five (5) copies of the approved Operating Manuals shall be provided to the COMPANY.

7.2	As-Built Records:

The CONTRACTOR shall provide a comprehensive set of as-built records, including the following:

-	General Arrangement Drawings.

-	Construction Drawings.

-	Logic diagrams (schematics).

-	Material test certificates to DIN 50049 3.1B.

-	Electrical test certificates.

-	Hydraulic test certificates.

-	Test data/curves.

Five (5) copies of all documents shall be supplied, both at approval and final issue stages.

8.0	SPARE PARTS REQUIREMENT:

The CONTRACTOR shall procure and supply spare parts for tow year’s operation as recommended by Manufacturer.