SCHEDULE SPS-4

PROJECT SPECIFICATION FOR TELECOMS EQUIPMEN

TABLE OF CONTENTS

1.0	SCOPE

2.0	CODES, STANDARDS & COMPANY SPECIFICATIONS.

3.0	GENERAL

4.0	OPTICAL LINE TERMINAL

5.0	DIGITAL BRANCHING EQUIPMENT

6.0	INTERFACES

7.0	EXISTING SYSTEM INTERFACE

1.0	SCOPE:

This specification is intended to define the minimum requirements for the supply of the telecommunication equipment for the ARAB GAS PIPE LINE - PHASE II

2.0	CODES, STANDARDS AND COMPANY SPECIFICATIONS

The telecommunication equipment shall comply with:

-	CCITT latest recommendations.

-	AFPC Specification, Exhibit-G.

3.0	GENERAL

3.1	Transmitters, receivers and auxiliary equipment shall be all solid state and designed according to state of the art techniques, including the optical line transceiver.

3.2 	Modular construction including printed circuit boards shall be employed to facilitate maintenance.

Transistor circuitry shall be fully protected against overload.

3.3	All modules shall be fitted with test points to facilitate monitoring of equipment parameters during the test and alignment.

3.4	Forced air cooling is prohibited and properly designed heat sinks shall be incorporated to provide convection cooling.

3.5	Transmission equipment shall use STM-1rate 155.52 Mb/S and suitable to be

Upgraded to STM-4 (622 Mb/s).

3.6	The system and all equipment shall be capable of operating in a tropical, desert climate, under the environmental conditions for temperature, humidity and wind as given below, as well as under heavy dust storms.

The range of environmental conditions is as follows:

Indoor equipment:

Standard performance 	-5 to 50 degree C. (all specifications met).

Relative humidity	10 % to 90 %

Operational ranges	-14 to 52 degree C.

Outdoor equipment:

Temperature range		-14 to +82 degree C.

Relative humidity		10% to 90%

Wind speed 			max. 160 km/hr.

EPUEPMENT SPESEFICATION

1- V-24/28 port card:

The card provides four separate standard DCE interfaces for connection to 48, 56, and 64 to 1920 kbps synchronous terminals.

The four separate interfaces are designed for point- to – point networking. as    four timeslots my be allocated to each card, each link may be transmitted in a particular timeslot, or be combined with one , tow or three more links into one timeslot, depending on the network topology and bit rate.

2- 6 subscriber port card:

The card supports six separate interfaces providing voice line connections to any extensions, videotext terminal; class 3 facsimile machines, and modes having a standard Z interface.

Each interface is the 2- wire type with standard 48 V powering and loop    disconnect signaling.

3- 6 exchange port card:

The card supports six separate interfaces providing connection to PABXs having

Standard Z interface.

Each interface is the 2- wire type with standard 48 V powering and loop disconnect

Signaling.

The subscriber code used is 1- bit signaling code.

4- 4-2048 kbps structured port card:

This card feature an HDB3 interface complying with the ITU-T recommendations

G.703 and G.704, which may support a single bit rate link or a multiplex of N X 64

Kbps channels.

Transmission performance monitoring is performed to supply common equipment

Units with errored block (blocks are groups of 2048 bits at 2 Mbps) information.

This procedure complies with the ITU-T recommendation G 704.

Depending on the particular operating environment, the (4-2048 kbps structured

port card ) may support the following modes of operation:

- Tributary operation:   TRANSMIC 2G, G.704/1920 kbps andG.704/1984

Kbps.

- Tributary operation:                  TRANSMIC 2G, G.736 – G.704/1920 kbps, G.736 – G.704/1984 kbps, G.732, international G.732.

The (4-2048 kbps structured port card) offers, in addition:

the virtual interface function which allows to connected several N X 64 kbps link of various origins but with a single destination on a 2 Mbps circuit,

- 2 Mbit/s path availability protection.

5-the system should be provided with ethernet port for transmitting all data between FMS's,BVS's and GCC, take note that V-24 data card will be as stand by way for data transmission.

3.7	The equipment shall be operated at-48 VDC (positive pole earth).

3.8	This specification basically defines the requirements for the optical fiber transmission equipment.

For general telecommunication equipment, auxiliaries and telephone cables, reference shall be made to schedule “HH”.

4.0	OPTICAL LINE TERMINAL

4.1	GENERAL:

The optical line terminal shall have such a design that on 155.52 Mb/s bit stream will be converted into a corresponding optical signal working at a 1550 nm wavelength.

The System shall be suitable to inter work with a second order digital multiplexer.

The optical line terminal equipment shall comply with the relevant CCITT recommendations:

G.702, G.703, G.704, and G.914.

The system shall be equipped with a double hetero junction laser module supplying the optical power into the optical fiber.

The equipment is designed for operation on optical fiber cable with high bandwidth optical fibers of the mono mode.

The optical receiver shall be a PIN diode. A fault tolerant line code shall be used.

4.2	SYSTEM SUPERVISION:

The telecommunication equipment shall be equipped with a supervisory system to ensure operation. This supervisory system shall be equipped with a micro processor based fault locating system which will continuously monitor the performance of the transmission equipment.

The supervising system shall work over the same optical fiber as the transmission channels and shall not make use of the multiplex transmission camme’s.

The supervising system shall provide for a central station configuration including VDU and data logging facilities enabling central monitoring of the transmission network.

It shall be possible to disconnect the 155.52 Mb/s input and output at the front of the line terminating units for measuring purposes.

The laser unit shall have an access point for measuring of the electrical input signal to the laser diode.

5.0	DIGITAL BRANCHING EQUIPMENT.

In order to drop/insert dedicated channels at the intermediate repeaters, digital branching equipment is foreseen.

The branching equipment related to the channel drop/insert performs the functions on a 2 MB/S signal, i.e. it may branch or insert one or several of the channels without de-multiplexing the main 2 MB/S signal.

Electrical characteristics:

2 MB/S interfaces:

Bit rate                                  2.48 Mbit/s ± 50   ppm

Ports                                     ITU-T G.703

Code                                     HDB3

Connector                              for HE5 – 37 – way socket connectors 16 + 5 separate I/O

75   connection	HE5 /BNC or 1.6 /5.6 adapter external to the sub-rack

155 MB/S interface

Bit rate                                  155.520 Mbit/s ± 20   ppm

Ports                                     ITU-T G.703, G.707, G.708, G.709

Code                                     CMI

Connector                              1.6/5.6 or BNC

OPTICAL:

Bit rate                                   155.520 Mbit/s ±15 ppm

Standard                                 ITU-T G.957/G.958

Optical fiber                           single mode (1550 nm, ITU-T G. 652)

Guaranteed attenuation          0.28 db

Typical range                        0-90 km

Connector                               FC/PC

Laser monitoring                    yes

Optical safety                         yes

Branching:

-	Channel branching.

Number of branched time slots up to30.

Number of omnibus channels	up to 63.

Number of branched 63 E1

These E1 to be drop/insert into channel level (64Kb/s, V24/28, Ethernet, 10Mb/s, 2 wire/4wire – with E&M).

6.0	INTERFACES.

6.1	All the ancillary equipment at each location shall be housed into a single front bay for floor installation or wall mounted cabinet according to the space required.

6.2	Frames shall be installed in the telecom. Buildings near the BVS’s, Intermediate stations, launcher, receiver, the Syrian-Turkish border and in the metering buildings at the FMS stations and equipped as follows:

-	Cable terminal.

-	Matching transformer.

-	Telephone termination.

-	Connection terminal boards etc.

-            Telephone devises.

7.0	EXISTING SYSTEM INTERFACE.

- The CONTRACTOR should interface with the existing system at Gas coordination center.