PEA APPENDIX 1

UNITS OF MEASUREMENT

1.	All units of measurement on drawings and documents shall be based on “Systems 	International Unites” (SI units) and their derivatives. For example:

QUANTITY UNIT SYMBOLS

(SI BASE UNITS)

Length					Meter			m

Mass					Kilogram		kg

Time					Second			s

Electric current			Ampere		A

Thermodynamic temperature		Kelvin			K

Amount of substance			Mole 			Mol

Luminous intensity			Candela		cd

(SI SUPPLEMENTARY UNITS)

Plane angle				Radian			rad

Solid angle				Steradian		sr

(SI DERIVED UNITS)

Frequency				Herz			Hz

Force					Newton		N

Pressure				Pascal			Pa

Energy					Joule			J

Power					Watt			W

Electric potential			Volt			V

Electric capacitance			Farad			F

Electric resistance 			Ohm			

Temperature				Celsius			C

Luminous flux				Lumen			lm

Illuminance				Lux			lx

2.	The following prefixes shall be used to indicate decimal multiples of the above 	units:

M					Mega			1,000,000

k					kilo			1,000

m					milli			0,001

					micro			0,000 001

Compound prefixes, and other multiples than the above, shall not be used. For instance, length can be expressed in m, mm, m or km: cm shall not be used.  For ease of reading, the choice of a decimal multiple shall result in a numerical value between 0.1 and 1,000.  For example, 0.00394 m shall be written as 3.94 mm, but not as 0.394 cm, nor as 3,940 m.

For engineering drawings, however length should be expressed in metres and 	millimetres not centimetres.

3 	Administration requires the following exceptions only to SI units to be used:

Pipeline diameters and flange sizes 	inches

OD tubing and fittings 	inches.

Pressure ratings 	ANSI stds

4.  	The following units shall be used in engineering documents (performance curves, 	data sheets etc.) and for scales of field and control room mounted instruments:

Density			kg/m3

Mass flow 			kg/s		kg/h

Volumetric flow 		m3 /s		m3 /h		m3 /d

l/s		l/h		l/d

ml/s		ml/h		ml/d

Volume, standard conditions	m3 (st)

(15 C, 1.01325 bar)

Volume flow, standard	m3 (st)/s	m3 (st)/h	m3 (st)/d

conditions

(15 C, 1.01325 bar)

Temperature			C

Pressure, absolute		mbar a		bar a 		kPa a		Mpa a

Pressure, gauge		mbar g		bar g		kPa g		MPa g

Pressure, differential		mbar		bar		kPa		MPa

Level				mm		m

Viscosity, dynamic		mm2/s

Viscosity, kinematic		mPa.s

G.O.R.				m3 /m3 (st)

Power				mW		W		kW		MW

Power apparent		kVA

Electric current		A

Voltage			mV		V		kV

Frequency			Hz

Force				N		kN

Energy				J		kJ		MJ

Vibration, amplitude		m		(peak-to-peak)

Vibration, velocity		mm/ s

Vibration, acceleration	mm/s2

Corrosion			m

Corrosion rate			mm/y

Rotary speed			rpm

Pump head, NPSH		m

Note: rpm, C, bar, y (year), h (hour) and d (day) are not strictly SI units but their use is permitted.

5. 	Oil field units

The following imperial oil field measuring units may be applied for gross throughput rates in facilities design where the RFQ so permits:

Gas flow (Standard Cubic Meter)		Scm/d		MScm/d	MMScm/d

Oil/water flow	(Barrel)			bpd		Mbpd

Standard conditions for the above units are defined as 60 F and 14.696 psia.

In the latter 1 barrel (b) is defined as 42 US Gallons for all liquids.