PEA APPENDIX 2

DRAWING NUMBERING SYSTEM

Numbering System for Drawing, Line Numbers and Equipment Tag Numbers.

###### Drawing Numbering

Drawing numbers shall be sequential according to the following numbering scheme;

XXXB - XX - XXX

¦           ¦       ¦

¦           ¦       ¦

¦           ¦      +----------   	Drawing number

¦          +----------------- 	Discipline function

+---------------------------	Contract number (A=Arab)

Discipline Function.

10 -	Process Flow Diagram/ Process & Instrument Diagram/plot plan

40 -	Civil/ Foundation

50 -	Piping

60 -	Electrical

70 -	Instrument

90 -	Structural

Line Numbers

Line numbers shall be numbered as follows

XXX-XX”-X-X-XXXX-XX X X

¦      ¦     ¦   ¦      ¦        ¦   ¦  ¦----------	Tracing Type

¦      ¦     ¦   ¦      ¦        ¦   ¦------------ 	Insulation Type

¦      ¦     ¦   ¦      ¦        ¦--------------- 	Insulation Thickness (mm) (if applicable)

¦      ¦     ¦   ¦      ¦ --------------------- 	Line unique number.

¦      ¦     ¦   +-------------------------- 	Piping Class.

¦      ¦     +----------------------------- 	Service Line.

¦     +-----------------------------------	Line Size (“).

+-----------------------------------------	Site Identifier (A-Arab)

Line Size (Imperial in inches)

Service Line

G	GAS

L	LIQUID

P	TWO PHASE

CD	CLOSED DRAIN

F	FLARE

V	VENT

UD	UTILITY DRAIN

CW	COOLING WATER

FW	FIRE WATER

PW	POTABLE WATER

SW	SEWER

UW 	UTILITY WATER

AI	INSTRUMENT AIR

AU	UTILITY AIR

GI	INSTRUMENT GAS

GU 	UTILITY GAS

HY	HYDRAULIC

PG	POWER GAS

BG	BLANKET GAS

DF	DIESEL FUEL

FG	FUEL GAS

N	NITROGEN

SG	STRIPPING GAS

LO	LUBE OIL

SO	SEAL OIL

C	CHEMICAL

GL	GLYCOL

K	CAUSTIC

PL	PIPELINE

EX	EXHAUST

AM	AMINE

Piping Class.

| SPECIFICATION SCHEDULES O & OO | SERVICE | ANSI CLASS |
| --- | --- | --- |
| A OR AA | Hydrocarbons, Glycol or Injection Water | 150 |
| AF | Flare, Pilot and Ignitor Lines | 150 |
| B OR BB | Hydrocarbons, Glycol or Injection Water | 300 |
| BW | Water Supply | 300 |
| C OR CC | Hydrocarbons, Glycol or Injection Water | 600 |
| D OR DD | Hydrocarbons, Glycol or Injection Water | 900 |
| E OR EE | Hydrocarbons, Glycol or Injection Water | 1500 |
| G | Fire, Utility Water, Starting Utility and Instrument Air | 150 |
| I | Glycol, Steam, Hot Water, Heating Oil | 300 |

Insulation Type

IA 	Acoustic Insulation

IC 	Cold Insulation

IE 	Electrical Trace

IF 	Thermal Fluid Trace

IH 	Heat Conservation

IS 	Safety Insulation

Tracing Type

E	ELECTRIC

G	HEAT MEDIUM

Equipment Tag Numbers

XX – XXX

‌‌ ‌|         ‌‌ ‌|

‌‌ ‌|         ‌‌ ‌|------------ Unique Equipment Number

‌‌ ‌|--------------------- Equipment Identifier

Tag numbers shall have the following pre-fixed Equipment Identifier;

B  	Buildings

C  	Compressor

D  	Dryer

E  	Exchanger

F  	Filter

FM 	Flotation Mixer or Skimmer

G  	Generator

H  	Heater

L  	Scraper Launcher

M  	Miscellaneous e.g. Mixer, Meter etc.

P  	Pump

S  	Sump / Drain Pit

T  	Tank

TM 	Tank Mixer

R  	Scraper Receiver

V  	Vessel

X  	Electrical

WH 	Wellhead

IM 	Inlet Manifold

CM 	Central Manifold

Z	Chemical Injection

Instrument Numbering

XXXX – XXX X

‌‌ ‌|          ‌‌ ‌|    ‌‌ ‌|-----------  Identifier for 2 or more identical instruments on the same equipment

‌‌ ‌|          ‌‌ ‌|----------------  Equipment Number to which Instrument is related

‌‌ ‌|--------------------------  Instrument Identifier (see instrument symbol identification)

Valve Numbering

V X X – XXX – XX

‌ ‌|  ‌ ‌|  ‌ ‌|        ‌ ‌|          ‌ ‌|-------------  Modifier to indicate special service

‌ ‌|  ‌ ‌|  ‌ ‌|        ‌ ‌|-----------------------  ANSI Rating

‌ ‌|  ‌ ‌|  ‌ ‌|-------------------------------- Connection Type

‌ ‌|  ‌ ‌|-----------------------------------  Valve Type

‌ ‌|--------------------------------------  Valve

Valve Type

- Ball

- Check

- Diaphragm

- Gate

- Needle

- Globe

- Butterfly

- 3 Way Valve

Connection Type

B-Butt weld

F-flanged

S-Socket welded

T-Threaded

Modifier

DBB-	Double Block and Bleed

FO-	Full Opening

SG-	Sour Gas

TC-	Through Conduit

H-       Hot